How to burn the firmware to flash

- Get the file all we need;

- Install the install_digilent.exe;

- Reboot the computer;

- Cope the burn_flash folder to the sd card;

- Insert the SD card into the module and connect to the computer through the debug port

- Find the uart port;

- Use the 2ed port as an console;

- Open the putty

- Click the open

- Type the end we will get an linux console;

- Enter commands

- Reboot

- Type when the countdown

- Type sf probe

- Type sf erase 0x0 0x2000000

- Type boot

- mount  /dev/mmcblk0p1  /mnt

- cd /mnt/burn_flash

- chmod +x autoconfig.sh

- ./autoconfig.sh

- Wait for complete;

- Enter the poweroff command

- Congratulations, you have completed 99% of the work

- Install the pluto driver Windows Drivers [Analog Devices Wiki]

- Connect the module use the otg port;

- Pluto/M2k Firmware Updates [Analog Devices Wiki]    Update the firmware I provided with UPDATE.BAT

- Wait for a moment

- Reconnect the usb.

- You will see an LED flashing;

Summary:

1. We use SD cards to burn down the basic parts

2. Using dfu to burn and write pluto firmware

Attention points:

When using dfu mode, unplug the SD card